# Design Note: IBIAS / ICMP Bias Network for LTC6811 isoSPI Configuration

Objective: justify the selection of RB1 and RB2 (R11 and R13 in schematic) that configure the isoSPI bias current IB and comparator threshold in the LTC6811-based BMS slave board.

## 1. Functional Background (From LTC6811 Datasheet)

The IBIAS pin provides approximately 2 V when the isoSPI interface is enabled. The bias resistors RB1 and RB2 program the isoSPI bias current IB. This current directly sets the isoSPI signaling currents.

The ICMP node (divider midpoint) sets the comparator reference. The comparator threshold is:

(Eq. 1)  V_TCMP = (1/2) · V_ICMP

The IP and IM drive currents are proportional to the programmed bias current:

(Eq. 2)  I_IP = I_IM = 20 · IB

Therefore, increasing IB increases the signaling current amplitude, which improves noise immunity but increases power consumption.

## 2. Selecting the Bias Current IB

The datasheet allows IB to be selected in the range of approximately 100 µA to 1 mA.

For most systems with cable lengths up to 50 m, IB = 0.5 mA is recommended as a compromise between power and noise immunity.

However, due to the electromagnetic interference (EMI) environment created by high-power inverters in this application, the design intentionally selects:

(Design Choice)  IB = 1 mA

Rationale: Increasing IB increases the isoSPI drive current:

(Eq. 3)  I_drive = 20 · IB

For IB = 1 mA → I_drive = 20 mA.
For IB = 0.5 mA → I_drive = 10 mA.
Doubling IB doubles signal amplitude, improving noise margin in high-EMI environments.

## 3. Programming IB with RB1 and RB2

The bias resistors RB1 and RB2 form a network from the 2 V IBIAS source to ground. The total resistance sets the bias current.

Assuming the 2 V IBIAS pin sources current through the series combination of RB1 and RB2:

(Eq. 4)  IB = V_IBIAS / (RB1 + RB2)

Given V_IBIAS ≈ 2 V and target IB = 1 mA:

(Eq. 5)  RB1 + RB2 = V_IBIAS / IB = 2 V / 1 mA = 2000 Ω

The chosen values are:

RB1 = 1.21 kΩ

RB2 = 806 Ω

Total = 2016 Ω ≈ 2 kΩ (within tolerance)

## 4. Setting the Comparator Threshold

The ICMP node voltage is set by the resistor divider:

(Eq. 6)  V_ICMP = V_IBIAS · (RB2 / (RB1 + RB2))

With RB1 = 1.21 kΩ and RB2 = 806 Ω:

V_ICMP = 2 V · (806 / 2016) ≈ 0.80 V

Comparator threshold:

(Eq. 7)  V_TCMP = (1/2) · V_ICMP ≈ 0.40 V

This threshold scales proportionally with IBIAS and maintains correct switching margins for the isoSPI receiver.

## 5. Power Dissipation Check

Worst-case resistor power (IB = 1 mA):

(Eq. 8)  P_total = V_IBIAS · IB = 2 V · 1 mA = 2 mW

Each resistor dissipates less than 2 mW, which is far below the rating of 0805 resistors (typically 125 mW), providing large safety margin.

## 6. Final Design Justification Summary

• IB selected at 1 mA (maximum recommended) to maximize noise immunity in high-EMI inverter environment.
• Total resistance chosen ≈ 2 kΩ to program IB ≈ 1 mA from 2 V IBIAS source.
• Divider ratio sets comparator threshold appropriately (V_TCMP ≈ 0.4 V).
• Power dissipation is negligible relative to component ratings.
• 0805 package selected for robustness and voltage rating margin.