# Design Note: Stack-Powered VREG (5 V) Linear Regulator Using Pass BJT + Series Zener (LTC6811 Slave Board, Cell 12 Top Cell)

This document justifies the choice of a simple linear regulator architecture (pass BJT controlled by the LTC6811 DRIVE pin) with a series zener dropper, and sizes the key components using first-principles math. The design goal is a reliable ~5 V rail (VREG) for the LTC6811 slave circuitry under pack min/max voltage, while minimizing complexity and maintaining good noise performance for cell-voltage measurement and isoSPI.

## 1. Requirements and Operating Conditions

Target rail: VREG ≈ 5.0 V (nominal).

Supply node: top of stack referenced to local slave ground (GND).

Top-of-stack voltage range (12S):

(Eq. 1)  V_IN = N_cells · V_cell

(Eq. 2)  V_IN,max = 12 · 4.2 V = 50.4 V

(Eq. 3)  V_IN,min ≈ 12 · 2.8 V = 33.6 V  (example minimum per-cell voltage)

Load current (worst-case peak): I_OUT,max ≈ 40 mA (LTC6811 peak + support circuitry).

## 2. Architecture Choice: Linear Pass Element vs Buck Converter

A switching buck converter would improve efficiency, but it substantially increases complexity and BOM: controller IC, inductor, diode/synchronous switches, layout constraints, EMI filtering, and additional capacitors. A linear regulator implemented with a pass BJT controlled by the DRIVE pin is simpler and inherently quiet.

Linear dissipation is:

(Eq. 4)  P_LOSS,linear = (V_IN − V_OUT) · I_OUT

Worst-case linear loss without a zener drop (V_OUT = 5 V, V_IN,max = 50.4 V, I_OUT,max = 40 mA):

(Eq. 5)  P_LOSS,noZ = (50.4 − 5.0) · 0.040 = 1.816 W ≈ 1.8 W

Over an example endurance window of 0.5 hr and 12 slave boards, energy loss is:

(Eq. 6)  E_loss,total ≈ P_LOSS,noZ · N_slaves · t

(Eq. 7)  E_loss,total ≈ 1.8 W · 12 · 0.5 hr = 10.8 Wh ≈ 11 Wh

This is typically a very small fraction of pack energy (order-of-magnitude ~0.1%), so the efficiency gain of a buck is not worth the added development and EMI complexity in this design context.

## 3. Circuit Description (Role of Each Component)

CELL_12_POS feeds the rail through a fuse (F5). A series zener diode (D2) drops a fixed voltage. A pass BJT (Q1) controlled by DRIVE regulates VREG near 5 V (linear regulation). C9 provides local decoupling. An LED (D7) with series resistor (R14) indicates VREG presence.

## 4. Core Regulator Math

Define: V_IN = input, V_Z = zener drop, VREG = output, I_OUT = load current.

Voltage budgeting:

(Eq. 8)  V_IN = V_Z + V_CE(Q1) + VREG

(Eq. 9)  V_CE(Q1) = V_IN − V_Z − VREG

Dissipation:

(Eq. 10) P_Z = V_Z · I_OUT

(Eq. 11) P_Q1 = (V_IN − V_Z − VREG) · I_OUT

(Eq. 12) P_total = (V_IN − VREG) · I_OUT

## 5. Zener Voltage Selection: Why ~25 V

The zener’s purpose is to reduce Q1 dissipation by dropping part of the input voltage. It must still leave sufficient headroom for regulation at the lowest pack voltage.

Regulation headroom condition (conservative):

(Eq. 13)  V_IN,min − V_Z ≥ VREG + V_CE,min

Assume V_CE,min ≈ 1 V →

(Eq. 14)  V_Z ≤ V_IN,min − VREG − 1 V

Using V_IN,min = 33.6 V and VREG = 5 V:

(Eq. 15)  V_Z ≤ 33.6 − 5 − 1 = 27.6 V

Therefore V_Z ≈ 25 V satisfies Eq. (15) with margin, so regulation remains possible even at low cell voltage.

## 6. Thermal/Dissipation at Maximum Pack Voltage

With V_Z = 25 V, V_IN,max = 50.4 V, VREG = 5 V, I_OUT,max = 40 mA:

(Eq. 16)  P_Z,max = 25 · 0.040 = 1.00 W

(Eq. 17)  V_CE,max = 50.4 − 25 − 5 = 20.4 V

(Eq. 18)  P_Q1,max = 20.4 · 0.040 = 0.816 W ≈ 0.8 W

The total loss is still ~1.8 W, but now the heat is shared between D2 and Q1 (improved thermal distribution).

## 7. Component Justification and Sizing

7.2 Series Zener (D2)

Chosen V_Z ≈ 25 V to satisfy low-voltage headroom (Eq. 15) and reduce Q1 dissipation (Eq. 18).

Power rating must exceed P_Z,max ≈ 1.0 W with margin; choose ≥2 W class or ensure thermal path supports continuous 1 W.

7.3 Pass BJT (Q1)

Select V_CE rating comfortably above V_IN,max (commonly 60–100 V class). Select thermal capability such that P_Q1,max ≈ 0.8 W is safe at worst-case ambient. Use: T_J ≈ T_A + P_Q1·θ_JA and keep T_J below device limit.

## 8. Verification at Minimum Pack Voltage

At V_IN,min = 33.6 V and V_Z = 25 V:

(Eq. 20)  V_headroom = V_IN,min − V_Z = 8.6 V

(Eq. 21)  V_CE = V_headroom − VREG = 8.6 − 5 = 3.6 V

Since 3.6 V > 1 V headroom assumption, the circuit can still regulate ~5 V at minimum pack voltage.