# BMS Shunt Current Measurement Design – Reflection

I designed the current measurement block for my 12‑series battery management system using a shunt resistor and the INA240 current sense amplifier. The goal of this design was to measure bidirectional pack current up to ±30 A reliably while ensuring that the amplifier output remained within the safe input range of the STM32F446 ADC.

## System Assumptions

• Battery configuration: 12S lithium pack (~50.4 V max)

• Maximum current range: ±30 A

• Shunt resistor value: 1 mΩ

• Current sense amplifier: INA240A2 (gain = 50 V/V)

• INA240 supply voltage: 3.3 V

• Reference midpoint: 1.65 V (half of 3.3 V)

• MCU ADC range: 0–3.3 V (STM32F446)

## Shunt Resistor Calculations

I selected a 1 mΩ shunt resistor as a compromise between power dissipation and measurement resolution.

Shunt voltage equation:

Vsense = I × R

At the maximum current:

Vsense,max = 30 A × 0.001 Ω = 0.03 V = 30 mV

Power dissipated in the shunt:

P = I²R

Pmax = 30² × 0.001 = 0.9 W

Although the theoretical dissipation is 0.9 W, I decided to use a shunt rated for at least 3 W to provide thermal margin and prevent drift due to heating.

## INA240 Output Calculation

The INA240 multiplies the shunt voltage by a fixed gain. I selected the INA240A2 variant which has a gain of 50.

Amplifier equation:

Vout = Vref + Gain × Vsense

Maximum output swing:

ΔVout = 50 × 30 mV = 1.5 V

With Vref = 1.65 V:

Vout,min = 1.65 − 1.5 = 0.15 V

Vout,max = 1.65 + 1.5 = 3.15 V

This range stays completely inside the STM32 ADC input limits (0–3.3 V), which was one of the main reasons I decided to power the INA240 from 3.3 V instead of 5 V.

## Firmware Current Conversion

Rearranging the amplifier equation gives the current measurement formula:

I = (Vout − Vref) / (Gain × Rshunt)

Substituting the constants:

I(A) = (Vout − 1.65) / 0.05

Which simplifies to:

I(A) = 20 × (Vout − 1.65)

Example values:

• Vout = 3.15 V → +30 A

• Vout = 1.65 V → 0 A

• Vout = 0.15 V → −30 A

## INA240 Connections

IN+  → system side of the shunt

IN−  → pack side of the shunt

VS   → 3.3 V

GND  → system ground

REF1 → GND

REF2 → 3.3 V

OUT  → STM32 ADC input (PB9)

## Decoupling and Filtering

I added a 0.1 µF decoupling capacitor between the VS and GND pins of the INA240 and placed it as close to the device as possible. This capacitor suppresses high‑frequency supply noise.

I also added a small RC filter on the output before the MCU ADC to reduce switching noise and stabilize ADC samples.

Filter components:

• 100 Ω series resistor between INA OUT and ADC pin

• 10 nF capacitor from ADC pin to ground

## Layout Considerations

The most important layout rule for this design is Kelvin sensing of the shunt resistor. I routed two dedicated sense traces directly from the shunt terminals to the INA240 IN+ and IN− pins. This prevents measurement error caused by voltage drop in the high‑current copper.

I also kept the high‑current path short and wide, and placed the INA240 close to the shunt to reduce noise pickup.

## Reflection

While working on this design, I realized that the biggest challenge in current sensing is not the equations but ensuring that the amplifier output stays within the ADC range of the microcontroller. Initially I considered powering the INA240 from 5 V with a 2.5 V reference midpoint, but that would have produced outputs up to 4 V, which exceeds the STM32 ADC limit.

After re‑evaluating the design, I switched to powering the INA240 from 3.3 V and using a 1.65 V midpoint. This change ensured that the full ±30 A current range could be measured without risking damage to the MCU.

I also learned how critical Kelvin connections are for shunt measurement accuracy. Even small voltage drops in the copper traces can introduce large errors when the signal being measured is only a few tens of millivolts.

Overall, this design process helped me better understand the trade‑offs between shunt resistance, amplifier gain, ADC range, and PCB layout in battery management systems.