# Slave Board Drive Circuit Design Reflection with Mathematical Justification

This document describes the mathematical and engineering reasoning behind the drive circuit used to power the LTC6811 on the slave board.

## 1. System Overview

The slave board is powered directly from the 12-cell lithium-ion stack rather than an external low-voltage battery. The maximum cell voltage is 4.2 V per cell.

Maximum stack voltage:

V_stack,max = 12 × 4.2 V = 50.4 V

The objective of the drive circuit is to convert this voltage to a regulated 5 V rail to power the LTC6811 and supporting analog components such as operational amplifiers.

## 2. Why Direct BJT Dropping Is Not Feasible

If a BJT were used alone to drop voltage from 50.4 V to 5 V, the voltage drop would be:

V_drop = 50.4 V − 5 V = 45.4 V

Power dissipation in the BJT would be:

P_BJT = V_drop × I_load

For example, assuming a conservative load current of 40 mA:

P_BJT = 45.4 V × 0.04 A = 1.816 W

Dissipating nearly 2 W in a small-signal transistor would lead to significant thermal stress, localized heating, and potential device failure.

## 3. Zener-Assisted Voltage Reduction

To reduce BJT dissipation, a 25 V Zener diode is placed in reverse breakdown before the BJT. This pre-drops a fixed voltage from the stack.

Voltage after Zener:

V_after_zener,max = 50.4 V − 25 V = 25.4 V

Now the BJT only needs to drop:

V_BJT_drop = 25.4 V − 5 V = 20.4 V

New BJT power dissipation:

P_BJT = 20.4 V × 0.04 A = 0.816 W

This reduces BJT thermal stress by more than 50%, making the design significantly safer and more reliable.

## 4. Minimum Voltage Condition

The Zener voltage must be chosen such that regulation is still possible when the cell stack is at minimum voltage. Assuming a minimum cell voltage of 2.8 V:

V_stack,min = 12 × 2.8 V = 33.6 V

Voltage after Zener at minimum stack voltage:

V_after_zener,min = 33.6 V − 25 V = 8.6 V

Since the BJT only needs to regulate down to 5 V, having 8.6 V available provides sufficient headroom for proper regulation even under low-cell conditions.

## 5. Additional Circuit Elements

D2 is a debug LED used for visual confirmation during board testing.

C3 is a bypass capacitor that provides local decoupling to reduce high-frequency noise and improve supply stability.

F1 is a protective fuse included for safety and compliance requirements. It protects against fault conditions such as short circuits or component failure.

## 6. Engineering Insight

The drive circuit reflects a deliberate tradeoff between simplicity, efficiency, thermal safety, and operating margin. The Zener-assisted drop strategy distributes power dissipation, maintains regulation across the full cell operating range, and prevents localized thermal overload in the BJT.